Face Trial 1

Appendix

Online conversion methods will convert mobile phone recorded videos which are in .mp4 format to .avi to be imported into ImageJ. However, the compression of the videos involved in this process is unsupported and the final video will not be able to be imported into ImageJ. (fijis built in avi reader is very old)

Therfore the best way to convert .mp4 to .avi is through ffmpeg. Ffmpeg is an a free and open-source software project that is a powerful suite of libraries and tools for handling multimedia files. It must be downloaded and opened in the command panel (for windows)

After downloaded and extracted ffmpeg needs to be added to the path. From

WIN+R

opening sysdm.cpl

advanced

environment variables

system variables

path

edit

new

add

C:\ffmpeg\bin

Ok

Test this by writing ffmpeg -version into the command prompt , if the version information is printed ffmpeg is ready to be used

The video to be converted should be added to the ‘bin’ file within the ffmpeg file. The command should then be ran within the bin folder by shift and right clicking open powershell window here inside the folder.

Once in the powershell command window print

ffmpeg -i "facetry2-ideal...Trim22411.mp4" -vcodec mjpeg -qscale:v 2 output_mjpeg.avi

where "facetry2-ideal...Trim22411.mp4" is the input video name, with .mp4 included.

Once converted the output video will also be in the bin file.

To export into imageJ, open Fiji go to file – open and choose output file , video will load as a stack (each frame becomes a slice)

Select ROI using rectangle or oval selection tool

OPTIONAL STABILIZE THE VIDEO

Plugins → Registration → StackReg

Rigid Body

Then split the colour channels, pulse is strongest in green

Image – Colour – Split Channels ,  use the green stack

With ROI still drawn

Image – Stacks – Plot Z axis Profile

Mean pixel intensity (y-axis) , frame number (x-axis)

This is the raw pulse waveform

Reduce noise by smoothing the signal

Process – Smooth

Process – FFT – Bandpass Filter

Large structures down 3-4 pixels

Small structures up to 0 pixels

Supress stripes OFF

Autoscale OFF

Go to List – File – Save As – pulse_data.txt

= A2 / 30 if fps is 30

IMPORT INTO MATLAB

Open heartrate code